Overview

The Orbit 7190g scanner continues the proven, superior 1D laser scanning performance of Orbit series scanners – but adds an area imager for seamless scanning of digital 1D, 2D and PDF codes in a single, convenient device. To speed retail checkouts, the Orbit 7190g scanner features a unique dual-modes design that is optimized for both the cashier and the customer – enabling seamless product barcode scanning and scanning of digital codes off customer smartphones.

Specifications

## MECHANICAL

Dimensions (L x W x H) 
108 mm x 103 mm x 148 mm (4.3 in x 4.1 in x 5.8 in)

Weight 
410 g (14.5 oz)

## ENVIRONMENTAL

Storage Temperature 
-40°C to 60°C (-40°F to 140°F)

Operating Temperature 
0°C to 40°C (32°F to 104°F)

Humidity 
5% to 95% relative humidity, non-condensing

Drop 
Designed to withstand 1.2 m (4 ft) drops

Environmental Sealing 
Sealed to resist airborne particulate contaminants

Light Levels
Laser: 4842 Lux
Imager: 100000 Lux

## ELECTRICAL

Input Voltage 
5 VDC ±0.25 V

Operating Power 
472 mA @ 5 V

Standby Power 
255 mA @ 5 V

Host System Interfaces 
USB, RS-232, Keyboard Wedge, IBM468xx (RS485)

EAS Features 
EAS with interlock and integrated RF EAS antenna (EAS model)

## SCAN PERFORMANCE

Scan Pattern 
Hybrid, Omnidirectional Laser (5 fields of 4 parallel lines) and Area Imager (640 x 480 pixel array)

Scan Speed
Omnidirectional: 1120 scan lines per second
FPS: 30

Scan Angle (Imager)
Horizontal: 40.0°
Vertical: 30.5°

Symbol Contrast 
35% minimum reflectance difference

Pitch, Skew
Laser: 60°, 60°
Imager: 60°, 70°

Decode Capability
Laser: Reads standard 1D, GS1 DataBar symbologies
Imager: Reads standard 1D, PDF and 2D symbologies

Warranty 
Three-year factory warranty

## TYPICAL PERFORMANCE*

LASER
5 MIL CODE 39: 0 mm – 50 mm (0 in – 2.0 in)
7.5 MIL CODE 39: 0 mm – 150 mm (0 in – 5.9 in)
10 MIL CODE 39: 0 mm – 220 mm (0 in – 8.7 in)
13 MIL UPC: 0 mm – 275 mm (0 in – 10.8 in)
26 MIL UPC-E: 0 mm – 300 mm (0 in – 11.8 in)

IMAGER
5 MIL CODE 39: 0 mm – 70 mm (0 in – 2.8 in)
13 MIL UPC: 0 mm – 245 mm (0 in – 9.6 in)
6.7 MIL PDF417: 0 mm – 70 mm (0 in – 2.8 in)
10 MIL DATAMATRIX: 0 mm – 75 mm (0 in – 3.0 in)
20 MIL QR: 0 mm – 200 mm (0 in – 7.9 in)
20 MIL C39: 0 mm – 327 mm (0 in – 12.9 in)

RESOLUTION 
Laser: 5 mil; Imager: 1D: 4mil, 2D: 6.7 mil

* Performance may be impacted by barcode quality and environmental conditions.