Overview

The Xenon™ 1900 series, featuring sixth-generation Adaptus™ area-imaging technology, has redefined the standard for handheld scanners. With its custom sensor that’s optimized for aggressive barcode scanning, the Xenon series offers industry-leading performance and reliability for a wide variety of data collection applications.

Specifications

## MECHANICAL

Dimensions (LxWxH)
1900/1900C - 1902/1902C Scanner: 104 mm x 71 mm x 160 mm (4.1 in x 2.8 in x 6.3 in) 
1902/1902C Base: 132 mm x 102 mm x 81 mm (5.2 in x 4 in x 3.2 in)

Weight
1900/1900C: 147 g (5.2 oz) 
1902/1902C Scanner: 214 g (7.5 oz) 
1902/1902C Base: 179 g (6.3 oz)

## ELECTRICAL

Input Voltage
1900/1900C : 4 VDC to 5.5 VDC 

OPERATING POWER
1900/1900C : 2.3 W (450mA @ 5 VDC) 
1902/1902C Scanner: N/A 
1902/1902C Base:

- Charging 5 W (1A @ 5 V)

- Non-Charging 0.5 W (0.1A @ 5 V)

Standby Power
1900/1900C : 0.45 W (90mA @ 5 VDC)

Battery
1902/1902C: 1800 mAh Li-ion (minimum)

Expected Operation Time
1902/1902Ci: 14 hours

Expected Charge Time
1902/1902C: 4.5 hours

Number of Scans
1902/1902C: 50,000 scans per charge

Host System Interfaces
1900/1900C : USB, Keyboard Wedge, RS232, IBM 46xx (RS485) 
1902/1902C Scanner: N/A 
1902/1902C Base: USB, Keyboard Wedge, RS232, IBM 46xx (RS485)

BLUETOOTH
1902/1902C Radio/Range: 2.4 to 2.5 GHz (ISM Band) Adaptive Frequency - Hopping Bluetooth v2.1; Class 2:10 m (33 ft) line of sight 
1902/1902C Data Rate (Transmission Rate): Up to 1 Mbit/s

## ENVIRONMENTAL

Operating Temperature
1900/1900C : 0°C to 50°C (32°F to 122°F) 
1902/1902C Scanner: 0°C to 50°C (32°F to 122°F) 
1902/1902C Base:

- Charging 5°C to 40°C (41°F to 104°F)

- Non-Charging 0°C to 50°C (32°F to 122°F)

Storage Temperature
-40°C to +70°C (-40°F to +158°F)

Humidity
0 to 95% relative humidity, non-condensing

Drop
1900 : Designed to withstand 50 1.8 m (6 ft) drops to concrete Ratchet stand: Designed to withstand 50 1.2 m (4 ft) drops to concrete on each of the faces 
1900C : Designed to withstand 50 1.8 m (6 ft) drops to concrete 
1902/1902C Scanner: Designed to withstand 50 1.8 m (6 ft) drops to concrete 
1902/1902C Base: Designed to withstand 50 1 m (3.3 ft) drops to concrete

Environmental Sealing
IP41

Light Levels
0 to 100,000 lux (9,290 foot-candles)

Ratchet Cycles
1900: The ratchet stand shall support 10,000 cycles (each cycle is one full movement of the scanner either up or down) without losing its ability to keep the scanner in any ratchet position

## SCAN PERFORMANCE

Scan Pattern
Area Image (838 x 640 pixel array)

Motion Tolerance
Up to 610 cm/s (240 in/s) for 13 mil UPC at optimal focus

Scan Angle
HD Focus: Horizontal 41.4°; Vertical: 32.2° 
SR Focus: Horizontal: 42.4°; Vertical: 33° 
1900/1902: 
ER Focus: Horizontal: 31.6°; Vertical: 24.4°

Print Contrast
20% minimum reflectance difference

Pitch, Skew
45°, 65°

Decode Capabilities
Reads standard 1D, PDF, 2D, Postal and OCR symbologies *Note: Decode capabilities dependent on kit configuration

Warranty
1900/1900C: 5 year factory warranty 
1902/1902C Scanner: 3 year factory warranty (Note: battery warranty is 1 year)